# HP Insight Management WBEM IPMI Record Log Provider Overview

| Description | The HP Insight Management Web-Based Enterprise Management (WBEM) Intelligent Platform Management Interface (IPMI) Record Log provider supports system logs available on HP servers. These logs are called the System Event Log (SEL) and the Forward Progress Log (FPL) and contain informational, warning, and critical event information about a server’s performance and sensors. This provider implements the following profiles and installs the necessary files: |  |  |
| --- | --- | --- | --- |
| Profile Name |  | Organization | Version |
| HP Record Log Profile (P00109 sample test edit) |  | HP WBEM TC | 1.0.0b |

| Requirements | For each hardware architecture listed, this provider requires the following distributions HP Integrity managed servers; SLES 10 and later RLEL 5.0 and later. HP ProLiant managed servers; SLES 11 and later RHEL 5.3 and later. |
| --- | --- |
| Release History | - Initial release with HP Insight Management WBEM Providers for Linux v2.0. |

## Setting Up the Provider

| Installing the Provider | There are no special installation instructions for this provider. It is installed by default as part of the HP Insight Management WBEM providers. |
| --- | --- |
| Configuring the Provider | This provider does not accept specific configuration adjustments beyond standard HP Insight Management WBEM support. |

## Using the Provider

Namespaces Supported by the Provider This provider returns instances in the root/hpq namespace.

| Schema Supported by the Provider |  |  |
| --- | --- | --- |
|  | This provider supports the following classes: - SMX_FPLRecordLog - SMX_FPLLogEntry - SMX_FPLLogManagesRecord - SMX_FPLUseOfLog - SMX_SELRecordLog - SMX_SELLogEntry - SMX_SELLogManagesRecord - SMX_SELRecordLogIndication - SMX_SELUseOfLog |  |

The tables in the following sections describe the properties of the supported classes. The classes are categorized by the class or superclass that defines the property, the first column is the property name (including type and units) and the second column describes how the provider determines the properties implementation. When the property implementation value is a number, the number given is the default behavior and the Managed Object Format interpretation is within parenthesis. If other values are returned, a problem is indicated.

Unless otherwise noted, all of the property implementation values given are for HP ProLiant and HP Integrity (cellular and non-cellular) systems. The location related properties and implementation values are determined based on the server type so they may differ.

### SMX_FPLRecordLog Class

The SMX_FPLRecordLog class implements the HP_CommonRecordLog class to model the server management record log.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | FPL record log |
| Description | Forward Progress Log |
| ElementName | FPL record log |
| CIM_ManagedSystemElement |  |
| Name | FPL record log |
| OperationalStatus | OperationalStatus[0] indicates whether the FPL log is in good health and accepting new FPL entries. 2 (OK), FPL is operating properly |
| StatusDescriptions | StatusDescriptions[0] will contain the following value corresponding to OperationalStatus[0]: OK |
| HealthState | 5 (OK), when OperationalStatus[0]=2 (OK) |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledState | 2 (Enabled) |
| RequestedState | 5 (Not Applicable) |
| EnabledDefault | 2 (Enabled) |
| CIM_Log |  |
| MaxNumberOfRecords | Number of records which will be greater than 0 |
| CurrentNumberOfRecords | Number of records in the FPL log (SMX_FPLLogEntry instances) |
| LogState | 2 (Normal) |
| OverwritePolicy | 2 (Wraps When Full) The FPL entries fill up available FPL non-volatile memory space until no room is left. The oldest log entries are then be deleted as needed to create room for new log entries. |
| CIM_RecordLog |  |
| InstanceID | HPQ:SMX_FPLRecordLog:1 |
| HP_CommonRecordLog |  |
| TimeOfLastChange | Time of last log change |

The following table lists implemented methods for the SMX_FPLRecordLog class.

| Method Name | Method Implementation |
| --- | --- |
| ClearLog | See the HP Record Log profile. |
| RepairAll | See the HP Record Log profile. |

### SMX_FPLLogEntry Class

The SMX_FPLLogEntry class implements the HP_CommonLogEntry class to model logs records within the FPL log.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | FPL record log entry <RecordID> Where: <RecordID> is the SMX_FPLLogEntry.RecordID for the event. |
| Description | Same as Caption. |
| ElementName | Same as Caption. |
| CIM_RecordForLog |  |
| CIM_LogEntry |  |
| InstanceID | HPQ:<opaque hexadecimal key> |
| LogInstanceID | SMX_FPLRecordLog.InstanceID |
| LogName | SMX_FPLRecordLog.Name |
| RecordID | FPL event number |
| HP_CommonLogEntry |  |
| RawData | FPL log event raw data array |
| MajorClass | 11 (FPL) |
| MinorClass | 2 (None) |
| SystemSerialNumber | System serial number string from SMBIOS. For example: USE4822L0H |
| SystemModel | System model number string from SMBIOS. For example: server rx6600 |
| SystemName | Computer system name. |
| LoggerType | 2 (Server) |

The following table lists implemented methods for the SMX_FPLLogEntry class.

| Method Name | Method Implementation |
| --- | --- |
| Repair | See the HP Record Log profile. |

### SMX_FPLLogManagesRecord Class

The SMX_FPLLogManagesRecord class implements the HP_LogManagesRecord class and associates instances of HP_RecordLog and HP_LogEntry.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_LogManagesRecord |  |
| HP_LogManagesRecord |  |
| Log | References SMX_FPLRecordLog |
| Record | References SMX_FPLLogEntry |

### SMX_FPLUseOfLog Class

The SMX_FPLUseOfLog class implements the HP_UseOfLog class and associates the instance of HP_ComputerSystem with the instance of HP_CommonRecordLog.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Dependency |  |
| CIM_UseOfLog |  |
| HP_UseOfLog |  |
| Antecedent | References SMX_FPLRecordLog |
| Dependent | References SMX_ComputerSystem |

### SMX_SELRecordLog Class

The SMX_SELRecordLog class implements the HP_CommonRecordLog class to model the server management record log.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | SEL record log |
| Description | System Event Log |
| ElementName | SEL record log |
| CIM_ManagedSystemElement |  |
| Name | SEL record log |
| OperationalStatus | OperationalStatus[0] indicates whether the SEL log is in good health and accepting new SEL entries. 2 (OK), SEL is operating properly |
| StatusDescriptions | StatusDescriptions[0] will contain the following value corresponding to OperationalStatus[0]: OK |
| HealthState | 5 (OK), when OperationalStatus[0]=2 (OK) |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledState | 2 (Enabled) |
| RequestedState | 5 (Not Applicable) |
| EnabledDefault | 2 (Enabled) |
| CIM_Log |  |
| MaxNumberOfRecords | Number of records available in NVRAM. This will be greater than 0 |
| CurrentNumberOfRecords | Number of records in the SEL log (not equal to the SMX_SELLogEntry instances as these instances are the sum of both SEL log entries and the SEL cache). |
| LogState | 2 (Normal) |
| OverwritePolicy | 2 (Wraps When Full) The SEL entries fill up available SEL non-volatile memory space until no room is left. The oldest log entries are then be deleted as needed to create room for new log entries. |
| CIM_RecordLog |  |
| InstanceID | HPQ:SMX_SELRecordLog:1 |
| HP_CommonRecordLog |  |
| TimeOfLastChange | Time of last log change. |

The following table lists implemented methods for the SMX_SELRecordLog class.

| Method Name | Method Implementation |
| --- | --- |
| ClearLog | See the HP Record Log profile. |
| RepairAll | See the HP Record Log profile. |

### SMX_SELLogEntry Class

The SMX_SELLogEntry class implements the HP_CommonLogEntry class to model logs records within the SEL log.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | SEL record log entry <RecordID> Where: RecordID is the SMX_SELLogEntry.RecordID for the event. |
| Description | Same as Caption. |
| ElementName | Same as Caption. |
| CIM_RecordForLog |  |
| CIM_LogEntry |  |
| InstanceID | HPQ:<opaque hexadecimal key> |
| LogInstanceID | SMX_SELRecordLog.InstanceID |
| LogName | SMX_SELRecordLog.Name |
| RecordID | SEL event number. |
| HP_CommonLogEntry |  |
| RawData | SEL log event raw data array |
| MajorClass | 12 (SEL) |
| MinorClass | 2 (None) |
| SystemSerialNumber | System serial number string from SMBIOS. For example: USE4822L0H |
| SystemModel | System model number string from SMBIOS. For example: server rx6600 |
| SystemName | Computer system name. |
| LoggerType | 2 (Server) |

The following table lists implemented methods for the SMX_SELLogEntry class.

| Method Name | Method Implementation |
| --- | --- |
| Repair | See the HP Record Log Profile. |

### SMX_SELLogManagesRecord Class

The SMX_SELLogManagesRecord class implements the HP_LogManagesRecord class and associates instances of HP_RecordLog and HP_LogEntry.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_LogManagesRecord |  |
| HP_LogManagesRecord |  |
| Log | References SMX_SELRecordLog |
| Record | References SMX_SELLogEntry |

### SMX_SELUseOfLog Class

The SMX_SELUseOfLog class implements the HP_UseOfLog class and associates the instance of HP_ComputerSystem with the instance of HP_CommonRecordLog.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Dependency |  |
| CIM_UseOfLog |  |
| HP_UseOfLog |  |
| Antecedent | References SMX_SELRecordLog |
| Dependent | References SMX_ComputerSystem |

## Provider Indications

### SMX_SELRecordLogIndication Class

The SMX_SELRecordLogIndication class implements and extends the HP_CommonRecordLogIndication class to Alert Clients to to new WBEM events found in the SEL Record log.

The following table describes the properties that are exposed for the SEL Record Log Provider indications that are implemented for server platforms, where available:

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication. |
| IndicationTime | Time of indication. |
| CIM_AlertIndication |  |
| EventTime | Time of the event or time of the indication if event time unknown. |
| SystemName | SMX_ComputerSystem.Name |
| SystemCreationClassName | SMX_ComputerSystem.CreationClassName |
| HP_AlertIndication |  |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 1 (Other) |
| Description | Long text description of the event. |
| EventCategory | Any of 30 Supported WBEM Event Categories: 0-9, 11,14,16,18, 20-21, 23,25,28, 33-34, 39, 40-43, 45-46 |
| EventID | Valid specific WBEM Event Number. |
| ProviderName | FPL_IndicationProvider |
| ProviderVersion | Provider Version in the format VV.UU.FF. For example: 02.03.10 |
| NetworkAddresses | Contains a list of all the IP addresses or hostname of the computer system generating the indication. |
| OSType | On Linux, 36 (Linux) |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>, <minor>, <build> |
| ProbableCause | 0 (Unknown) |
| ProbableCauseDescription | Short text describing cause if known. |
| RecommendedActions | If action exists, this array will contain text values of human readable strings for potential resolution. |
| Summary | Short Text describing the SEL Record Log Event. |
| SystemFirmwareVersion | Array of firmware versions of the computer system generating the indication. |
| SystemSerialNumber | Serial number of the computer system generating the indication. |
| SystemProductID | Product ID of the computer system generating the indication. |
| SystemModel | Model name of the computer system generating the indication. |
| SystemGUID | Platform GUID of the computer system generating the indication. |
| SystemVirtualUUID[] | If Synergy is enabled, this will contain the logical UUID for the system. |
| SystemVirtualSerialNumber[] | If Synergy is enabled, this will contain the logical Serial Number for the system. |
| EnclosureName | HP_BladeEnclosureCS.Name |
| RackName | Rack name, if one exists |
| RackUUID | Rack Unique Identifier, if one exists |
| BladeName | HP_ComputerSystem.Name, if BladeSystem |
| BladeBay | HP_BladeCSLocation.LocationInformation[0], if BladeSystem |
| HP_CommonRecordLogIndication |  |
| PerceivedSeverity | 0 (Unknown) 3 (Degraded/Warning) 6 (Critical) |
| RecordLogInstanceID | Opaque Record Log key. |
| LogEntryInstanceIDs | Array of length 1 containing an opaque Log Entry key. |
| SMX_SELRecordLogIndication |  |
| RawData | Array if 14 bytes of raw event data. |
| VariableNames | Indexed arrays VariableNames[0] per VariableValues[0] per VariableTypes[0], any or all of the following values: “IPMIEventHex” “DataFieldTypeName” “DecodedDataFiled” “ReportingEntityID” “ReportingEntityFullName” “IPMIEventID” “SWID” |
| VariableTypes | 1 |
| VariableValues | For VariableNames “IPMIEventHex”: This will be hex string representing the IPMIEvent For VariableNames “DataFieldTypeName”: This will contain text Physical Location For VariableNames “DecodedDataFiled”: “Physical Location = “ followed by HWPhysicalLocation For VariableNames “ReportingEntityID”: Value will contain hex string representing the ReportingEntity For VariableNames “ReportingEntityFullName”: Value will be NA For VariableNames “IPMIEventID”: Value will contain hex string representation of IPMIEventID For VariableNames “SWID”: Value will contain hex string representation of SystemGUID |
| HWPhysicalLocation | 8-Byte Physical Location blob represented as a string For example: “00-ff-ff-ff-ff-05-ff-64” |