July 28, 2026

Dear Premeire/PCI II ED customer,

Using the Premeire/PCI II ED the following  wattages can be attained when the system is fully powered down, or asleep.

The system configuration is as follows:

Plato 90Mhz base board PBA#541288-004

Cirrus 5434 Video/ 1MB VRAM

Quantum Q540AT IDE hdd

Teac 3.5" Floppy

BIOS 1.00.10.AX1

32MB memory

DOS 6.0

A.C. outlet power of 120V

Options used in taking measurements during testing:

NEC 5FGe

P54C B1 or B3 stepping

Astec 3450 Power supply or Power Win PW200APS

| Monitor | P54C stepping | Power Supply | Watts |
| --- | --- | --- | --- |
| 5 FGe | B1 | Astec | 61.9 |
| Not measured | B1 | Astec | 36.0 |
| 5 FGe | B3 | Astec | 55.6 |
| Not measured | B3 | Astec | 30.4 |
| 5 FGe | B3 | PowerWin | 55 |
| Not measured | B3 | PowerWin | 29.2 |

To obtain these measurements the BIOS setup utility was used to change the Advanced power management setting to enabled, Power down was set to 10 minutes, and the HOT KEY combination was set to Ctl-Alt-K.

If ten minutes, or the amount of time set by the user, elapses without user intervention the system will power down, however, if the HOT KEY sequence is pressed then 1 minute will elapse regardless of the time-out setting of 10 minutes.

In the above results a better reading was obtained by using a different power supply. Efficiency of the PowerWin supply helped gain about 1 watt. Also A.C. Power being too high will affect your final wattage.

OPSD Technical Marketing